## 6(a) Connecting Arduino with a mobile device using Bluetooth

Components: Arduino Uno R3 only. In the simulation the Serial Monitor input box is the phone.

Procedure

- Start the simulation and open the Serial Monitor.

- Type any character, such as H, in the input box and press Send.

- The Arduino replies with the character it received, confirming two-way communication.

Program

Stream &BT = Serial;   // real HC-05: see Appendix A

void setup() {

Serial.begin(9600);

BT.println("Arduino connected. Send any character.");

}

void loop() {

if (BT.available()) {

char c = BT.read();

if (c == '\n' || c == '\r') return;

BT.print("Received: ");

BT.println(c);

}

}

Expected output

Arduino connected. Send any character.

Received: H

## 6(b) Control two actuators (LED and servo) using Bluetooth

Components: Arduino Uno R3, LED, 220 ohm resistor, micro servo.

Circuit connections

| Component pin | Connect to |
| --- | --- |
| LED anode via 220 ohm | D8 |
| LED cathode | GND |
| Servo signal | D9 |
| Servo power / ground | 5V / GND |

Note: Commands: A = LED on, a = LED off, S = servo to 90 degrees, s = servo to 0 degrees.

#include <Servo.h>

Stream &BT = Serial;   // real HC-05: see Appendix A

Servo arm;

const int LED = 8;

void setup() {

Serial.begin(9600);

pinMode(LED, OUTPUT);

arm.attach(9);

arm.write(0);

BT.println("A=LED on  a=LED off  S=Servo 90  s=Servo 0");

}

void loop() {

if (BT.available()) {

char c = BT.read();

switch (c) {

case 'A': digitalWrite(LED, HIGH); BT.println("LED ON"); break;

case 'a': digitalWrite(LED, LOW);  BT.println("LED OFF"); break;

case 'S': arm.write(90); BT.println("SERVO 90"); break;

case 's': arm.write(0);  BT.println("SERVO 0"); break;

case '\n': case '\r': break;

default: BT.println("Unknown command");

}

}

}

Expected output

Type A and the LED glows and LED ON is printed.

Type S and the servo moves to 90 degrees and SERVO 90 is printed.

Result: Thus Experiment 6 (Connecting Android Phone with Arduino (Bluetooth)) was designed and simulated successfully in Tinkercad Circuits.

# Experiment 7: Integrating Ethernet Shield: Socket Communication

Aim: To read data from sensors and send it to a requesting client using socket-style request and response communication.

In socket communication a client sends a request to a server identified by an IP address and port, and the server replies. An Arduino Ethernet shield (W5100 chip, SPI interface) lets the Arduino act as the server. Tinkercad has no Ethernet shield, so the Serial Monitor plays the client and the Arduino plays the server, using the same request and response pattern. The code for a real Ethernet shield web server is in Appendix A.

## Sensor data server

Components: Arduino Uno R3, TMP36 temperature sensor, Photoresistor with 10 kilo-ohm resistor.

Circuit connections

| Component pin | Connect to |
| --- | --- |
| TMP36 left / right / middle | 5V / GND / A0 |
| Photoresistor leg 1 | 5V |
| Photoresistor leg 2 (junction) | A1 and one end of 10k resistor |
| 10k resistor other end | GND |

Note: In the Serial Monitor type a request such as GET TEMP and press Send.

Program

const int TEMP = A0;

const int LDR = A1;

float readTemp() {

return (analogRead(TEMP) * 5.0 / 1023.0 - 0.5) * 100.0;

}

int readLight() {

return map(analogRead(LDR), 0, 1023, 0, 100);

}

void setup() {

Serial.begin(9600);

Serial.setTimeout(200);

Serial.println("Server ready. Requests: GET TEMP | GET LIGHT | GET ALL");

}

void loop() {

if (Serial.available()) {

String req = Serial.readStringUntil('\n');

req.trim();

req.toUpperCase();

if (req == "GET TEMP") {

Serial.print("200 OK TEMP=");

Serial.print(readTemp(), 1);

Serial.println(" C");

} else if (req == "GET LIGHT") {

Serial.print("200 OK LIGHT=");

Serial.print(readLight());

Serial.println(" %");

} else if (req == "GET ALL") {

Serial.print("200 OK TEMP=");

Serial.print(readTemp(), 1);

Serial.print(" C, LIGHT=");

Serial.print(readLight());

Serial.println(" %");

} else if (req.length() > 0) {

Serial.println("400 BAD REQUEST");

}

}

}

Expected output

Server ready. Requests: GET TEMP | GET LIGHT | GET ALL

GET TEMP  ->  200 OK TEMP=25.0 C

HELLO  ->  400 BAD REQUEST

Result: Thus Experiment 7 (Integrating Ethernet Shield: Socket Communication) was designed and simulated successfully in Tinkercad Circuits.

# Experiment 8: Creating Mobile App

Aim: To create a mobile app interface to control an actuator, and to control electronic devices from anywhere over the Internet.

A mobile app controlling a device simply sends short text commands. Tinkercad cannot run a phone app or reach the Internet, so the Serial Monitor is used as the app and each command is exactly what the app would send. To build the real app, use MIT App Inventor: add a BluetoothClient, a button for each command, and call BluetoothClient.SendText with the command text on each click. For remote control, the app sends the command to a cloud service and the board fetches it, as modelled in 8(b).

## 8(a) Mobile app control of an actuator

Components: Arduino Uno R3, LED, 220 ohm resistor, DC motor (fan), NPN transistor, 1 kilo-ohm resistor, diode 1N4007.

Circuit connections

| Component pin | Connect to |
| --- | --- |
| LED anode via 220 ohm / cathode | D8 / GND |
| D6 via 1k resistor | Transistor base |
| Transistor emitter | GND |
| Transistor collector | Motor terminal 1 |
| Motor terminal 2 | 5V |
| Diode across motor | Cathode to 5V, anode to collector |

Note: Check the transistor pin labels by hovering over them. Commands: LED ON, LED OFF, FAN ON, FAN OFF, FAN 50 (speed in percent).

Program

const int LED = 8;

const int FAN = 6;

void setup() {

Serial.begin(9600);

Serial.setTimeout(200);

pinMode(LED, OUTPUT);

pinMode(FAN, OUTPUT);

Serial.println("LED ON/OFF, FAN ON/OFF, FAN <0-100>");

}

void loop() {

if (Serial.available()) {

String cmd = Serial.readStringUntil('\n');

cmd.trim();

cmd.toUpperCase();

if (cmd == "LED ON") {

digitalWrite(LED, HIGH); Serial.println("OK LED ON");

} else if (cmd == "LED OFF") {

digitalWrite(LED, LOW); Serial.println("OK LED OFF");

} else if (cmd == "FAN ON") {

analogWrite(FAN, 255); Serial.println("OK FAN ON");

} else if (cmd == "FAN OFF") {

analogWrite(FAN, 0); Serial.println("OK FAN OFF");

} else if (cmd.startsWith("FAN ")) {

int p = constrain(cmd.substring(4).toInt(), 0, 100);

analogWrite(FAN, map(p, 0, 100, 0, 255));

Serial.print("OK FAN SPEED ");

Serial.print(p);

Serial.println("%");

} else if (cmd.length() > 0) {

Serial.println("ERROR unknown command");

}

}

}

Expected output

LED ON  ->  OK LED ON

FAN 50  ->  OK FAN SPEED 50%

## 8(b) Control devices from anywhere using Internet and mobile app

Components: Arduino Uno R3, LED (lamp), 220 ohm resistor, piezo buzzer (siren).

Circuit connections

| Component pin | Connect to |
| --- | --- |
| Lamp LED anode via 220 ohm / cathode | D8 / GND |
| Piezo positive / negative | D7 / GND |

Note: A remote message carries a secret key so that only the owner can control the devices. Send for example KEY=1234; LAMP=ON; SIREN=OFF

Program

const String SECRET = "1234";

const int LAMP = 8;

const int SIREN = 7;

bool lampOn = false;

bool sirenOn = false;

String getValue(String msg, String name) {

int s = msg.indexOf(name + "=");

if (s < 0) return "";

s += name.length() + 1;

int e = msg.indexOf(';', s);

if (e < 0) e = msg.length();

return msg.substring(s, e);

}

void setup() {

Serial.begin(9600);

Serial.setTimeout(200);

pinMode(LAMP, OUTPUT);

Serial.println("Format: KEY=1234;LAMP=ON;SIREN=OFF");

}

void loop() {

if (!Serial.available()) return;

String msg = Serial.readStringUntil('\n');

msg.trim();

msg.toUpperCase();

if (msg.length() == 0) return;

if (getValue(msg, "KEY") != SECRET) {

Serial.println("ACCESS DENIED");

return;

}

String lamp = getValue(msg, "LAMP");

String siren = getValue(msg, "SIREN");

if (lamp == "ON") lampOn = true;

if (lamp == "OFF") lampOn = false;

if (siren == "ON") sirenOn = true;

if (siren == "OFF") sirenOn = false;

digitalWrite(LAMP, lampOn);

if (sirenOn) tone(SIREN, 1500); else noTone(SIREN);

Serial.print("ACK LAMP=");

Serial.print(lampOn ? "ON" : "OFF");

Serial.print(" SIREN=");

Serial.println(sirenOn ? "ON" : "OFF");

}

Expected output

KEY=1234;LAMP=ON  ->  ACK LAMP=ON SIREN=OFF

KEY=0000;LAMP=ON  ->  ACCESS DENIED

Result: Thus Experiment 8 (Creating Mobile App) was designed and simulated successfully in Tinkercad Circuits.

# Experiment 9: Interfacing Cloud

Aim: To push environmental sensor data from Arduino to a cloud server, and to control an actuator through the cloud.

ThingSpeak is a free IoT cloud platform. A write API key lets a device add data to a channel through an HTTP address, and a read request returns the latest value of a field. Tinkercad has no network access, so the Arduino prints the exact ThingSpeak address it would send. You then act as the network link by pasting that address into a browser. Create a free ThingSpeak account and a channel with Field 1 (temperature), Field 2 (light), Field 3 (humidity), and note the Write API Key.

## 9(a) Push sensor data to the cloud server

Components: Arduino Uno R3, TMP36, Photoresistor with 10 kilo-ohm resistor, potentiometer (stands in for a humidity sensor, as Tinkercad has no DHT11).

Circuit connections

| Component pin | Connect to |
| --- | --- |
| TMP36 left / right / middle | 5V / GND / A0 |
| LDR leg 1 / junction / 10k to GND | 5V / A1 / GND |
| Potentiometer outer pins / wiper | 5V and GND / A2 |

Procedure

- Replace YOUR_WRITE_API_KEY in the code with your key and start the simulation.

- Every 15 seconds (the ThingSpeak minimum) a full address is printed in the Serial Monitor.

- Copy that line and open it in a browser. A number such as 1 is returned and the data appears in your channel charts.

Program

const char* API_KEY = "YOUR_WRITE_API_KEY";

const unsigned long INTERVAL = 15000;

unsigned long lastSend = 0;

bool first = true;

void setup() {

Serial.begin(9600);

}

void loop() {

if (first || millis() - lastSend >= INTERVAL) {

first = false;

lastSend = millis();

float tempC = (analogRead(A0) * 5.0 / 1023.0 - 0.5) * 100.0;

int light = map(analogRead(A1), 0, 1023, 0, 100);

int hum = map(analogRead(A2), 0, 1023, 0, 100);

Serial.print("https://api.thingspeak.com/update?api_key=");

Serial.print(API_KEY);

Serial.print("&field1=");

Serial.print(tempC, 1);

Serial.print("&field2=");

Serial.print(light);

Serial.print("&field3=");

Serial.println(hum);

}

}

Expected output

https://api.thingspeak.com/update?api_key=YOUR_WRITE_API_KEY&field1=25.0&field2=48&field3=60

## 9(b) Control an actuator through the cloud

Components: Arduino Uno R3, LED, 220 ohm resistor.

Circuit connections

| Component pin | Connect to |
| --- | --- |
| LED anode via 220 ohm / cathode | D9 / GND |

Procedure

- In the ThingSpeak channel, set Field 4 to a brightness value from 0 to 100 (open the update address with &field4=75).

- Read it back in a browser: https://api.thingspeak.com/channels/YOUR_CHANNEL_ID/fields/4/last.txt (private channels need &api_key=READ_KEY). Check the ThingSpeak documentation if the address format has changed.

- Type the number returned into the Serial Monitor. The LED brightness follows the cloud value.

Program

const int LED = 9;

void setup() {

Serial.begin(9600);

pinMode(LED, OUTPUT);

Serial.println("Enter cloud value (0-100):");

}

void loop() {

while (Serial.available() && !isDigit(Serial.peek())) Serial.read();

if (Serial.available()) {

int v = constrain(Serial.parseInt(), 0, 100);

int pwm = map(v, 0, 100, 0, 255);

analogWrite(LED, pwm);

Serial.print("Actuator set to ");

Serial.print(v);

Serial.print("% (PWM=");

Serial.print(pwm);

Serial.println(")");

}

}

Expected output

75  ->  Actuator set to 75% (PWM=191)

Result: Thus Experiment 9 (Interfacing Cloud) was designed and simulated successfully in Tinkercad Circuits.

# Experiment 10: Data Analysis and Visualization

Aim: To collect sensor data, analyse it (minimum, maximum, average) and visualize it using a spreadsheet or cloud visualization service.

## Log, analyse and plot sensor data

Components: Arduino Uno R3, TMP36, Photoresistor with 10 kilo-ohm resistor.

Circuit connections

| Component pin | Connect to |
| --- | --- |
| TMP36 left / right / middle | 5V / GND / A0 |
| LDR leg 1 / junction / 10k to GND | 5V / A1 / GND |

Procedure

- Start the simulation. While the 20 samples are collected (one every 1.5 s), click the TMP36 and change its temperature, and change the light level.

- When the summary prints, select all Serial Monitor text and copy it.

- Paste into Excel or Google Sheets, use Data > Split text to columns with comma as separator, then Insert > Chart > Line chart.

- Optional: use the ThingSpeak data from Experiment 9 and its built-in charts or MATLAB Visualizations app.

Program

const int N = 20;

float temp[N];

int light[N];

void setup() {

Serial.begin(9600);

Serial.println("sample,temp_C,light_pct");

for (int i = 0; i < N; i++) {

temp[i] = (analogRead(A0) * 5.0 / 1023.0 - 0.5) * 100.0;

light[i] = map(analogRead(A1), 0, 1023, 0, 100);

Serial.print(i + 1);

Serial.print(",");

Serial.print(temp[i], 1);

Serial.print(",");

Serial.println(light[i]);

delay(1500);

}

float mn = temp[0], mx = temp[0], sum = 0;

for (int i = 0; i < N; i++) {

if (temp[i] < mn) mn = temp[i];

if (temp[i] > mx) mx = temp[i];

sum += temp[i];

}

Serial.println("---- Temperature summary ----");

Serial.print("Min: ");  Serial.println(mn, 1);

Serial.print("Max: ");  Serial.println(mx, 1);

Serial.print("Mean: "); Serial.println(sum / N, 1);

}

void loop() {

}

Expected output

sample,temp_C,light_pct

1,25.0,48

2,25.4,47

...

---- Temperature summary ----

Min: 24.6

Max: 33.2

Mean: 28.1

Result: Thus Experiment 10 (Data Analysis and Visualization) was designed and simulated successfully in Tinkercad Circuits.